CS4163 - Advanced Networking, Virtualization and Cloud Computing

Practical ICA – (Assessment) 01
Task 02 - IP Addressing and VLAN Segmentation

01.Network Topology Overview

The network consists of a hierarchical design with:

- Core Layer: Two routers (R1 and R2)

- Aggregation/Distribution Layer: Three switches (S5, S6, S7)

- Access Layer: Four switches (S1-S4) connecting eight end devices (PC1-PC8)

Logical separation is implemented using VLANs for Management (VLAN 10) and Guest Users (VLAN 20).

02.IP Addressing Scheme

VLAN Segmentation

| VLAN | Purpose | Subnet | Gateway | Usable Range |
| --- | --- | --- | --- | --- |
| 10 | Management | 192.168.10.0/24 | 192.168.10.1 | 192.168.10.1 - 192.168.10.254 |
| 10 | Management | 192.168.30.0/24 | 192.168.30.1 | 192.168.30.1 - 192.168.30.254 |
| 10 | Management | 192.168.50.0/24 | 192.168.50.1 | 192.168.50.1 - 192.168.50.254 |
| 20 | Guest | 192.168.20.0/24 | 192.168.20.1 | 192.168.20.1 - 192.168.20.254 |
| 20 | Guest | 192.168.40.0/24 | 192.168.40.1 | 192.168.40.1 - 192.168.40.254 |
| 20 | Guest | 192.168.60.0/24 | 192.168.60.1 | 192.168.60.1 - 192.168.60.254 |

Inter-Router Connection

- Subnet: 192.168.100.0/30

- R1 (Serial0/3/0): 192.168.100.1

- R2 (Serial0/3/0): 192.168.100.2

End Device IP Assignments

| Device | VLAN | IP Address | Gateway | Connected Switch |
| --- | --- | --- | --- | --- |
| PC1 | 10 | 192.168.10.10 | 192.168.10.1 | S1 |
| PC2 | 20 | 192.168.20.10 | 192.168.20.1 | S1 |
| PC3 | 10 | 192.168.10.11 | 192.168.10.1 | S2 |
| PC4 | 20 | 192.168.20.11 | 192.168.20.1 | S2 |
| PC5 | 10 | 192.168.30.10 | 192.168.30.1 | S3 |
| PC6 | 20 | 192.168.40.10 | 192.168.40.1 | S3 |
| PC7 | 10 | 192.168.50.10 | 192.168.50.1 | S4 |
| PC8 | 20 | 192.168.60.10 | 192.168.60.1 | S4 |

03.Router Configurations

Router R1

interface GigabitEthernet0/2.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface GigabitEthernet0/2.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface Serial0/3/0

ip address 192.168.100.1 255.255.255.252

Router R2

interface GigabitEthernet0/1.10

encapsulation dot1Q 10

ip address 192.168.30.1 255.255.255.0

interface GigabitEthernet0/1.20

encapsulation dot1Q 20

ip address 192.168.40.1 255.255.255.0

interface GigabitEthernet0/2.10

encapsulation dot1Q 10

ip address 192.168.50.1 255.255.255.0

interface GigabitEthernet0/2.20

encapsulation dot1Q 20

ip address 192.168.60.1 255.255.255.0

interface Serial0/3/0

ip address 192.168.100.2 255.255.255.252

Switch Configurations (Example: Switch S1)

vlan 10

name Management

vlan 20

name Guest

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

interface FastEthernet0/2

switchport mode access

switchport access vlan 20

interface FastEthernet0/24

switchport mode trunk

switchport trunk allowed vlan 10,20

04.Testing & Verification

Routing Check:

VLAN Verification:

Ping Tests: